Template No.

CEMILAC_SYSGP_SOFT_13

SAFTEY OF FLIGHT TEST PROCEDURE

for <LRU/SYSTEM Name>

for

<Platform Name>

Issue/Rev No: 01/00

Date of Release: 8 Feb 2025

| <DESIGN AGENCY LOGO> |  | Document No. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Issue No./ Rev No. : | <00X> |  | Issue Date : |  | <DD/MM/YYYY> |
|  |  | Copy No. : | 01 of N |  | No. of Pages : |  | < total no .of pages > |
|  |  | Document Classification : |  | Secret Confidential Restricted Unrestricted |  |  |  |
| Title: |  |  |  |  |  | Project/System : |  |
| SAFTEY OF FLIGHT TEST PROCEDURE of<LRU/Module Name> for <Platform name> |  |  |  |  |  | <System/Project Name> |  |
|  |  |  |  |  |  | LRU/System Part No. |  |
|  |  |  |  |  |  | <No.> |  |
|  |  |  |  |  |  | Critical Level |  |
|  |  |  |  |  |  | <A/B/C/D/E> |  |
|  | Name & Designation |  |  |  |  | Signature |  |
| Prepared By | <Design Rep Name>, < Designation> <Agency Name> |  |  |  |  |  |  |
| Reviewed By | <Project Leader Name>, <Designation> <Agency Name> <AWG/QA HOD Name>, <Designation> <Agency Name> |  |  |  |  |  |  |
| Approved for Release By | <Project Leader Name>, <Designation> <Design Agency> <Officer_Name>, <Designation> RCMA <Name> |  |  |  |  |  |  |
| <Design Firm Name & Address> |  |  |  |  |  |  |  |

Disclaimer:

This document is a guidance document. Applicable section / table rows may be considered. Any additional details may be added. Any not applicable section/ table rows may be deleted. The template is very general and vary with process to process followed by Development Agency.  The document may be fine-tuned with the TAA for finalization.

Document Copy Distribution List

| Copy No. | DESIGNATION OF THE COPY HOLDER | ORGANISATION/DEPARTMENT |
| --- | --- | --- |
| 01 | Design Group Head | Design Agency |
| 02 | RD | RCMA |
| 03 | RD | ORDAQA |
| 04 | HOD | QC/ AWG, Design agency |
| 05 | Coordinator | Customer |

Amendment History

| Issue No. | Issue Date | Brief Description of Amendment | Change Request Ref. | Affected Pages | Affected Section | Change Effective From |
| --- | --- | --- | --- | --- | --- | --- |
| 001 |  | Initial Issue | NA | NA |  | All |
| 002 |  |  |  |  |  | Unit sl.no. <aaa> or Hw mod xx/ Sw ver x.x |

TABLE OF CONTENTS

1	Introduction	3

2	Purpose and Scope	3

3	System Description	3

4	Applicable Documents	3

5	Standards Reference	3

6	SOF Test Applicability	4

6.1	SOF Test Sequence	10

7	SOF test procedure:	11

7.1	Physical (Initial Visual) Examination	11

7.2	COTS Screening Test	11

7.2.1	High Temperature storage (stabilization bake) test	11

7.2.2	Thermal shock Test	12

7.2.3	Power Burn-In	12

7.3	Environment Stress Screening (ESS) Tests:	12

7.3.1	Pre-Thermal Vibration:	12

7.3.2	Thermal Cycling	13

7.3.3	Post-Thermal Vibration:	14

7.4	Initial Ambient checks	14

7.5	Continuous Run	14

7.6	Power Supply Tests	15

7.6.1	Transfer operation test:	15

7.6.2	Reverse polarity protection test:	15

7.6.3	Emergency operation test:	15

7.6.4	Steady State Normal and Abnormal Voltage tests:	16

7.6.5	Over voltage surge (Transient) test:	16

7.6.6	Under voltage surge (Transient) test:	16

7.6.7	Power failure test:	16

7.6.8	Voltage distortion Spectrum	17

7.6.9	Ripple test	17

7.6.10	Starting voltage transient	17

7.7	EMI-EMC Tests	17

7.8	Vibration tests	18

7.8.1	Initial Resonance Search	18

7.8.2	Endurance test	19

7.8.3	Final Resonance check	21

7.9	CATH	21

7.10	Shock	23

7.11	Acceleration Test	23

7.12	SHOCK (Crash Safety)	24

7.13	Rapid Decompression:	24

7.14	Explosive Atmosphere Test:	25

7.15	Gunfire Vibration	26

7.16	Final Ambient check:	26

ANNEXURE-I: Test sheets	29

ANNEXURE – II Compliance Report	30

List of Acronyms:

Give all the Acronyms used in this specification Alphabetically

# Introduction

Brief introduction to the overall functions of the system. (Not more than 1 page)

# Purpose and Scope

This document establishes the Safety of flight Test Procedure for evaluating the performance   of <System Name> <Part No.> for <platform>.

The various stakeholders in the project are as follows:

| Design Agency | <DLRL, RCI, HAL, BARC etc> |
| --- | --- |
| Development Agency | <HAL, BEL, ECIL etc> |
| Manufacturing Agency | <HAL, BEL, ECIL, BDL, ELIORA etc> |
| Acquirer | <ADA, RWRDC, ARDC etc> |
| User | <IAF, IN, Army Aviation, Coast Guard etc> |
| Certification agency | <CEMILAC, RCMA(Hyd) etc> |
| Production Quality Control Agency | <ORDAQA, MSQAA, RAQAS etc> |

# System Description

<Give a brief overview of system, project, for which this SOF testing is applicable. If FTP is not available for this system, describe the test setup covering simulators/ ATE/ Function generator / Testjigs/Oscilloscopes/Test Equipments connected to UUT that shall be used to verify the specifications and functionality of the UUT. Indicate the test setup in a figure>.

# Applicable Documents

<All the applicable documents of the system such as Approved Tech spec, ICD, FTP, SOFT, MDI, BOM, aircraft environmental map etc., along with the document number, issue number and date of approval to be given here>

# Standards Reference

<All the related standards like MIL, DO, JSS Standards and CEMILAC Directives etc.,  on which the tests are based,  should be given here>

- The CEMILAC Directive no : Ref No: CEMILAC / 5390 / AW / 4 / Directives, Issue – 01, Dated: 13 Feb 2015

# SOF Test Applicability

| Sl. No | TEST AND STANDARD | SEVERITY | Remarks |
| --- | --- | --- | --- |
|  | Physical Inspection | Weight, Dimensions and compliance of unit with respect to engineering standards and applicable drawings. |  |
| - 2 | COTS screening on PCBs - PCB and UUT words are interchangeable Ref : CEMILAC/5390/1 Dated 10-01-2014. Sec 2.2, 2.4 and 2.6 | - High temperature storage (stabilization bake) At +85°C bake the PCB - Thermal Shock Stabilized low temp -40°C for 30 min. Stabilized High temp +85°C for 30 min. 2 min transfer operation 1cycle. 10 such cycles - Power Burn in test At maximum rated operating temp PCB shall be maintained with power ON for 48Hrs. If failure occurs, replace the components with those which have undergone bake and thermal shock. | 1) 24 Hrs Duration After test UUT testing at Ambient. 2) UUT OFF condition If any components fail, the baking and thermal shock should be repeated on component. 3) Last 3 Hrs should be defect free. In case failure observed during 46/ 47/48 Hrs. expose PCB for further 3Hrs. |
| - 5 | ESS | - Pre-vibration - Thermal Cycling ( 10 cycles) - Post vibration | 1) Pre and post vibration duration is 5min. 2) The last 3 cycles of thermal cycling should be defect free. 3) Pre and post vibration duration is 5min. |
| - 3 | Initial Ambient Checks | FTP at nominal voltage |  |
| - 4 | Continuous Run | Verification of Unit operation throughout the period at ambient temperature. | 4 hrs |
| - 6 @ | Power Supply Variation MIL-STD-704F Table IV Fig 13, 15 (DC) Table I Fig. 3,5,6 (AC) Fig. 14 (DC) Fig. 4 (AC) Sec 5.3.2.3 Sec 5.1 | DC; AC. a) Normal Steady State LDC 102 22 – 29 V b)Voltage Distortion LDC 103 Table – II c)Total Ripple LDC 104 Table II d)Normal Voltage transient. LDC-105 LDC -105-III Test conditions AA-RR e)Transfer operation: LDC 201 Table-II f) Abnormal Steady State: LDC-301 Table –I & II Min : 30minutes Operation g) Over voltage surge: 50V for 0.05s Reduce to 31.5V in 7.7s 31.5V till 100 s LDC-302 Table –IV (AAA to FFF) h)Under voltage surge: 0V for 7 sec 20V till 100 sec LDC-302 Table –IV (GGG to LLL); 108 - 118V & 393 - 407 Hz. SAC 102 SAC 106, 107 & 108 SAC 104 & 105 Voltage and frequency modulation SAC 109 & 110 Voltage and frequency transients SAC 201 Table-II A-L 100 - 125V & 380 - 420 Hz. SAC 301 Table III 180V for 0.05s Reduce to 124.5V in 6.9s 125V till 100 s SAC 302 Table III AA-GG 0V for 7 sec 100V till 100 sec SAC 302 Table III AA-GG, HH-NN. I) Combined voltage surge: SAC 302 Table III LDC-302 Table –IV OO condition. (LLL to NNN) J )Emergency Operation: SAC 401 16V (DC)** Table-II LDC-401 Table –II k) Starting voltage transient LDC-501 Table-III Test condition AA l) Power Failure: SAC 601 Table LDC-601-I and II Table-II A-to-D m) Reverse polarity SAC 603 LDC-602 Table-I | a)Functional checks (at least 30 min) b) Not less than 5min operation at each frequency. c)Not less than 30 min at each test condition. d)The transients are from 29 to 50 at high end and to 18 in low end. e) # continued operation without reset, ensured based on user req. f) LDC-301 Test cases 30 min operation in min. g) LDC-302 Test cases repeated 5 times. h) LDC-302 Test cases repeated 5 times. I) LDC-302 Test cases repeated 5 times. j) ** If required by user. 12V DC operation for 15 min. during engine start. k) Test to be repeated 5 times. m) Phase reversal should not cause damage |
| - 7 | Vibration Ref: CEMILAC / 5390 / AW / 4 / Directives, Issue – 01, Dated: 13 Feb 2015 Section 2.5.7.1.1 | - Resonance search: * 5 Hz to 500Hz and back to 5Hz @0.5g sweep at 1 octave/min. Resonance noted if response is 2 times the stimulus. ** 5 Hz to 2KHz and back to 5Hz @0.5g sweep at 1 octave/min. Resonance noted if response is 2 times the stimulus. - Endurance - Rotary wing follow, Figure-4 profile. - Fixed wing propeller A/c, Figure-5 profile. - Fixed wing Jet Aircraft, Figure-6 profile. - Final Resonance search Final resonance search (similar to Initial Resonance search) shall be carried out after endurance test | - Unit OFF *Rotary wing ** Fixed wing Record the resonance frequencies and they should not coincide with the platform fundamental and its harmonics. b) 15min per axis (Unit ON condition) c)The final resonance should be done even there is no resonance observed in initial search. If any deviation noticed in resonance frequencies, corrective action should be taken. |
| - 8 | Combined Altitude Temperature Humidity (CATH) Ref: CEMILAC / 5390 / AW / 4 / Directives, Issue – 01, Dated: 13 Feb 2015 Section 2.5.7.2 | 30 min ramp, ambient to TSL OFF 2 Hr soak@ TSL OFF 10 min ramp to TOL OFF 2Hr soak@ TOL OFF 30 min soak@ TOL ON 30 min soak@ TOL * ON 30 min ramp to Altitude ON 30 min @ Max Altitude ON 30 min @ Max Altitude *ON 30 min ramp to GND Altitude ON 30 min Ramp from TOL to +430C And RH from Ambient to 95%RH ON 1Hr +430C 95%RH (*last 30 min) *ON 30 min Ramp from +430C to TOH And RH from 95% to Ambient 1Hr @ TOH and RH 30% * ON 10 min ramp from TOH to TSH OFF 2Hr soak @ TSH OFF 10 min ramp from TSH to TOH OFF 1Hr Soak @ TOH OFF 1Hr Soak @ TOH(*last 30 min) *ON 30 min ramp to Max Altitude ON 1Hr @ TOH and Max Altitude *ON 30 Min ramp from TOH to Ambient and Max Altitude to GND Altitude ON | Each cycle of appr. 15Hr 30 min duration. - cycle. *Performance check In last 30 min of Hr. Refer: Figure-7 Note: - The condition of TOH and low pressure can be removed based on the platform requirement. - If CATH is done then tests 14,15, 16 & 17 need not be done. TSH-High Storage Temp. TSL-Low Storage Temp. TOH-High Operating Temp. TOL-Low Operating Temp. |
| - 9 | Functional Shock | 20g sawtooth/15g half sine , 11mSec Pulse width | 3 shocks in each of 6 directions. Unit ON. |
| - 10 | Acceleration | Struct -A/c Heli Fore 3.0g 4.0g Aft 9.0g 4.0g Up 13.5g 10.5g Down 4.5g 4.5g Lat(L) 6.0g 6.0g Lat(R) 6.0g 6.0g | The Unit should be in OFF condition. 1 min along each direction This test can be carried out on mock up unit |
| - 11 | Shock (Crash Safety) Ref: CEMILAC / 5390 / AW / 4 / Directives, Issue – 01, Dated: 13 Feb 2015 Section 2.5.10.1 | 40g saw tooth/30g half sine 11mSec Pulse width | 3 shocks in each of 6 directions. Unit OFF. This test can be done on mock up unit. |
| - 12 | Rapid Decompression MIL-STD-810G sec 4.5.4 Page 505.5-6 | Temp=Ambient, Altitude=Ground Adjust chamber pressure to Altitude=2.5Km Reduce the pressure to Altitude=Ceiling Alt. In not more than 15 seconds, Hold for 10 min Come to ambient conditions with pressure rate 10m/s and temperature rate 3°C/min. | Hold at Max Alt for 10 Min. Unit ON |
| - 13 | Explosive Atmosphere | N-hexane reagent grade fuel to be used. 3.8% fuel by volume in test atmosphere. (Air to vapour ration of 8.33 by weight) Altitude : (maximum ceiling altitude) or 40000 ft + 2000 ft. reduce at 100 Mts/ min to Ambient ground altitude. Temperature: 71 ºC (Highest operating temp) Verify the explosiveness of the vapour mixture by trying to ignite a sample in another chamber or use calibrated explosive gas meter. Attempt operation of the equipment at 40000 ft. Note the altitude at start of correct operation. At ground altitude conduct performance check. | An explosive test shall be carried out to determine the ability of the equipment to either operate in fuel – air atmosphere without causing ignition and / or for verifying containment of an explosive or burning reaction of encased unit |
| - 14 | Gunfire Shock | As per distance 0-180 cm, 2.2 – 0.034 g2/Hz | 7 min/axis. Unit ON. This can be carried out on Mock-up. |
| - 15 | High Temperature Storage cum operation | 6 Hr Soak @ Ambient OFF 3Hr ramp from Ambient to TOH OFF 2Hr Soak @ TOH OFF 2Hr30 min ramp TOH to TSH OFF 2Hr Soak @ TSH OFF 1Hr 30 min Ramp TSH to TOH OFF 2Hr Soak @ TOH ON 3Hr Soak @ TOH ( Last 30 min ) *ON 3Hr ramp from TOH to Ambient ON | 24 Hr cycle 3 such cycles. TSH-High Storage Temp. TOH-High Operating Temp. |
| - 16 | Low Temperature Storage cum operation | Ambient ON 30c/Minute ramp from Ambient to TSL OFF 2Hr Soak @ TSL OFF 30c/Minute ramp TSL to TOL OFF 1Hr Soak @ TOL OFF 1Hr Soak @ TOL ON 30c/Minute ramp TOL to Ambient ON | 1 cycle. TSL-Low Storage Temp. TOL-Low Operating Temp. |
| - 17 | Humidity | 30 0C to 60 0C in 2 Hrs with 95% RH Soak at +60 0C for 6 Hrs with 95% RH (This is the most severe humid condition) Mil-std 810G sec 2.3.2 c) +60 0C to +300C in 8 Hrs with 85% RH Soak at +30 0C for 8 Hrs with 95% RH | 3 cycles. Unit OFF. Unit ON during last 30 min of the last cycle. |
| - 18 | Low Pressure | Ambient Alt pressure (performance Check) switch OFF the unit. Ramp from Ambient to Max Alt pressure @ 10 min/sec. 2Hr @ Max Alt pressure *ON Ramp from Max Alt to Ambient pressure @ 10 min/sec | 1 cycle. *Performance check during last 30 min. |
| - 19@ | EMI/EMC MIL-STD-461F | CE101, 102,106 RE101,102, 103 (Either CE106 or RE103, as applicable) | Unit ON. |
| - 20 | Final Integration | Integration of the Unit with external interfaces and verification of functional requirements. |  |
| - 21 | SW IV & V | Verification of S/W by independent team as per applicable Standard. |  |

Note: If CATH test is carried out, High temperature, Humidity, Low temperature and altitude tests need not be done.

## SOF Test Sequence

The SOF test clearance consists of the following tests:

- Initial visual examination

- Measurement of weight and physical parameters

- COTS screening test

- Environmental Stress Screening (ESS)

- Initial Ambient checks

- Continuous run  test

- Environmental and EMI/EMC tests are in the order as shown in the table below.

| Sl. no. | Test | Preferable sequence to carry out the test |
| --- | --- | --- |
|  | Power supply variation | Before all other, since component changes are expected |
|  | EMI/EMC |  |
|  | Vibration | After Power supply variation |
|  | Gunfire shock | After Vibration, mechanical and thermal shock |
|  | Acceleration | After high temp |
|  | Crash safety | After high temp |
|  | High Temp | After dynamic tests |
|  | Low temp | After dynamic tests |
|  | Low Pressure | After dynamic tests |
|  | CATH | Before Humidity |
|  | Humidity | After dynamic tests. |
|  | Explosive Atmosphere | After fungus, humidity, salt fog |
|  | Rapid decompression | After climatic tests |
|  | Functional shock | After vibration and thermal tests |

The procedure for each of the above tests is indicated in the section 7.0. Qualification test clearance shall be obtained based on the compilation of test report and preparation of compliance chart.

# SOF test procedure:

## Physical (Initial Visual) Examination

- The Unit shall be examined along with components and modules to determine the compliance with requirements of Engineering Standards and applicable drawings as per MDI document.

- The equipment shall be inspected for type/part number, serial number etc.

- The Unit and sub-modules shall be examined visually and any damage or fault noted should be reflected in the physical inspection report. This may include defects in construction, presence of foreign bodies, moisture, dust etc., and corrosion of materials/ finish distortion or mechanical imperfection.

- The weight and dimensions shall be measured and recorded as specified in the Technical specifications.

Any item, which does not meet this requirement, shall be rejected and recorded.

## COTS Screening Test

The following tests are carried out as per “Screening Procedure for COTS components”, “Appendix A” to CEMILAC/5390/1 Dated: 10th Jan 2004” PART B.

### High Temperature storage (stabilization bake) test

Temperature: +85°C± 5°C

Duration:  24 Hours

Power: OFF

Test Procedure:  Check the performance of the module in standalone mode. Keep the module in a thermal chamber. Set the temperature of the chamber to +85°C and bake it for 24 Hours without energizing the module. Check the performance of the module after the bake test as per Annexure <give test sheet reference>

### Thermal shock Test

Low Temperature: -40°C± 5°C Duration: 30 minutes

High Temperature: +85°C± 5°C Duration: 30 minutes

Number of cycles: 10

Transfer time: 2 minutes

Operation: OFF condition

Test Procedure: Place the module in a chamber which is maintained at a temperature of +85°C and keep it for 30 minutes, change the module to a cold chamber which is maintained at a temperature of -40°C and keep it for 30 minutes. Repeat the process 10 times. Check the performance of the module after the thermal shock test as per <give test sheet reference>

### Power Burn-In

Temperature: The highest operating temperature TOH of UUT.

Duration: 48Hrs

In case of component failure, the components to be replaced with the components which have undergone baking and thermal shock. If the failure occurs during 46th /47th /48th Hrs, then the repaired unit is subjected to 3Hrs extra after completion of 48Hrs to confirm the adequacy of the repair.

## Environment Stress Screening (ESS) Tests:

### Pre-Thermal Vibration:

The pre thermal vibration is to be carried out in all the three axes as per following specifications and profile in figure 1.

| Frequency | Level |
| --- | --- |
| 20 Hz to 80 Hz | 3 db/Octave rise ( + 3 dB/Oct) |
| 80 Hz to 350 Hz | 0.04 g² /Hz |
| 50 Hz to 2 K Hz | 3 dB/Octave roll off ( -3 dB/Oct) |

Duration: 5 Min/ Axis

Figure 1: ESS Vibration profile

Test setup: The UUT is mounted on the vibration with similar fixture as in the platform.

Test Procedure:

Fix the mounting plate on the vibration stand. Fix the UUT on the plate with the help of suitable tools. Switch ON the equipment and Apply random vibration for 5 min in 3 axes with equipment in ON condition. Verify and record parameters in test sheets <give test sheet reference >

### Thermal Cycling

Severity: -40˚C to +65˚C as per Fig (Thermal Cycling profile)

Test setup: The UUT shall be kept inside the temperature chamber.

Test Procedure and performance:

Thermal cycling consists of 10 nos. of COLD-HOT cycles as per figure 2. The last three cycles should be defect free. The procedure for thermal cycling is as follows.

- The equipment shall be kept in calibrated temperature chamber.

- With unit in OFF condition, raise the chamber temperature to +65oC.

- Soak the unit to this temperature for 60 minutes.

- Lower the chamber temperature to –40oC.

- Soak the unit to this temperature for 60 minutes.

- Switch ON the unit.

- Monitor and record parameters.

- Raise the chamber temperature to +65oC.

- Soak the unit to this temperature for 60 minutes.

- Monitor and record parameters.

- Switch off the unit.

Above steps (d) to (k) constitute one cycle. Remaining cycles have to be carried out		 same as above.

(l)Rate of change of temperature: During thermal cycling, the rate of change of temperature shall be at the rate of min 5oC/ minute.

Verify and record parameters in test sheets <give test sheet reference>

Figure 2: Thermal cycle profile

### Post-Thermal Vibration:

Same as Sec 7.3.1.

## Initial Ambient checks

The equipment shall be tested for complete performance / functional checks exercising the software residing in the equipment at nominal power supply voltages at ambient temperature. Any defects observed during inspection / performance checks shall be recorded and necessary corrective action initiated to eliminate the same.

## Continuous Run

Temperature: Ambient temperature

Duration:  4 Hours

Power: ON

Test Procedure:

The equipment shall be kept powered ‘ON’ continuously for 4 hours at ambient temperature with nominal power supply voltage. Performance Checks are to be carried out during the test. On completion of burn-in, the equipment shall be subjected to visual examination and Check the performance of the module and record the parameters as per < give test sheet reference >

Note-2: The 4 hours duration has been arrived at by considering the approximate maximum trial duration of single flight of aircrafts and helicopters. This may be tailored for 1.5 times the maximum mission duration.

Check the performance of the module at 5th hours period and record the parameters as per < give test sheet reference >

## Power Supply Tests

Power supply tests are as per MIL-STD-704F.

### Transfer operation test:

During transfer operation test,

- Set supply voltage to 28V DC followed by

- Switch Power supply to zero volts for a period of 50msec followed by switch back to 28V.

<Specify whether the unit is expected to continue operation during the power supply micro-cut or reset-and-restart is allowed>. The transfer of operation voltage test must be performed as per the MIL-STD-704-8 LDC201.

### Reverse polarity protection test:

During the reverse polarity protection test, reverse the negative and positive connections of the 28 V power supply to the unit. The unit shall not be damaged. Check the performance after correcting the connections. The reverse polarity test must be performed as per the Mil-Std 704-8 LDC 602.

### Emergency operation test:

Set the power supply at 16VDC (Emergency condition) and carry out the selected functional checks at extreme voltages as per FTP/ATP for at least 10 minutes and record the parameters as per <give test sheet reference>. The emergency operation test must be performed as per the MIL-STD-704-8 LDC401.

### Steady State Normal and Abnormal Voltage tests:

1. Connect the UUT and the power supplies for the readiness of the test.

2. Set the supply voltage to 22VDC and carry out the selected functional checks as per <give test sheet reference> at extreme voltages as per FTP/ATP for at least 30 minutes. Repeat the test with 29VDC. The steady state normal voltage test must be performed as per the MIL-STD-704-8 LDC102. The normal voltage transients should be carried as per LDC 105.

3. Set the supply voltage to 20VDC and carry out the selected functional checks as per <give test sheet reference> at extreme voltages  as per FTP/ATP for at least 30 minutes. Repeat the test with 31.5VDC. The steady state abnormal voltage test must be performed as per the MIL-STD-704-8 LDC301.

### Over voltage surge (Transient) test:

During over voltage test vary the supply voltage from 28VDC to 50 VDC. Hold at 50 V for 50 ms, bring it back to 31.5V in 7.7s and hold it at 31.5V as shown in figure 3. Carry out the selected functional checks at steady state voltage as per FTP/ATP and record the desired parameters as per <give test sheet reference>. The over voltage surge test must be performed as per the MIL-STD-704-8 LDC302.

### Under voltage surge (Transient) test:

During Under voltage test vary the supply voltage from 28VDC to 0V within less than 1 ms, and hold it for 7sec and in less than 1 ms bring it to 20VDC as shown in the figure 3. Unit shall not be damaged. Carry out the selected functional checks at extreme voltages as per FTP/ATP and record the desired parameters. The under voltage surge test must be performed as per the MIL-STD-704-8 LDC302.

### Power failure test:

The power failure test must be conducted to see the unit functionality under failure conditions. The test must be performed as per the MIL-STD-704-8 LDC601. Figure 3

TABLE LDC601-II. Test conditions for power failures.

| Test Condition | Duration of Power Failure |
| --- | --- |
| A | 100 msec |
| B | 500 msec |
| C | 3 seconds |
| D | 7 seconds |

Figure 3. Under-voltage and Over-voltage test profile.

### Voltage distortion Spectrum

This is to verify whether the equipment operates and maintains specified performance when subjected to voltage distortion of frequencies and amplitudes. The test must be performed as per Mil-Std 704-8 LDC 103.

### Ripple test

This is to verify whether the equipment operates and maintains specified performance when subjected to voltage having ripples. The test must be performed as per Mil-Std 704-8 LDC 104.

### Starting voltage transient

This is to verify whether the equipment operates and maintains specified performance when the unit is started and transients are expected. The test must be performed as per Mil-Std 704-8 LDC 501.

## EMI-EMC Tests

The following EMI/EMC measurements shall be as part of full qualification tests. There shall be no repetition of these tests on subsequent models of the equipment, unless the equipment is redesigned or major modification is carried out.

The list of EMI/EMC tests to be conducted on the unit are given in Table 1 as per MIL-STD-461F.

<The tests sl.no. 1, 3, 5, 6, 7, 8, 9, 13, 15, 16 and 18 are applicable selectively. Depending on the applicability, these tests may be removed from the table>

| Sl. no. | Test | Applied to | Frequency Range |
| --- | --- | --- | --- |
|  | CE101 | Power leads | 30 Hz to 10 kHz |
|  | CE102 | Power leads | 10 kHz to 10 MHz |
|  | CE106 * | Antenna Terminals | 10 kHz to 40 GHz |
|  | RE101 | Magnetic field | 30 Hz to 100 kHz |
|  | RE102 | Electric field | 10 kHz to 18 GHz |
|  | RE103 * | Spurious and harmonics | 10 kHz to 40 GHz |

Table 1: EMI/ EMC applicable tests

* - Either CE106 or RE103 may be carried out as applicable

## Vibration tests

This test shall be carried out to verify the capability of equipment to meet the vibration stresses. This test consists of three parts:

- Initial Resonance Test

- Endurance Vibration Test

- Final Resonance Test

### Initial Resonance Search

Initial Resonance test shall be carried out as per Table-2. After completion of this test carry out the performance checks.

Table-2: Resonance Tests

| Equipment mounted on Rotary Wing Platform | Resonance Search at 0.5g from 5 Hz to 500 Hz with equipment in 'OFF' condition shall be carried out. The test will be conducted with the test item mounted as is in the platform. Record the resonance frequencies (ratio > 2) observed. These frequencies should not coincide with the platform fundamental frequencies and their harmonics. |
| --- | --- |
| Equipment mounted on Fixed Wing Aircraft | Resonance search at 0.5g from 5 Hz to 2000 Hz with equipment in ‘OFF’ condition shall be carried out. The test will be conducted with the test item mounted as is in the platform. Record the resonance frequencies (ratio > 2) observed. These frequencies should not coincide with the platform fundamental frequencies and their harmonics. |

### Endurance test

Vibration endurance test shall be 15 minutes per axis in all three axes.

- For Rotary wing aircraft the profile shall be as per Figure-4. The values of W0, W1, fx, A1...A4 shall be derived from TABLE 514.5C-IV of Mil-STD-810F.

- For Fixed wing Propeller aircraft the profile shall be as per Figure-5. The values of L0, fx, shall be derived from Table 514.5C-II.of Mil-STD-810F.

- For Fixed wing Jet aircraft the profile shall be as per Figure-6.

The equipment shall be kept ON during the endurance vibration test with performance checks. On completion of the vibration test the complete performance checks shall be carried out

Acceleration Spectral Density – g2/Hz

W1

Log Frequency in Hz

10

f1

f2

f3

f4

100

Figure – 4: Endurance Vibration Profile for Rotary Wing Aircraft

Peak Acceleration – g

W0

300

ft

A1

A2

A3

A4

Acceleration Spectral Density – g2/Hz

L0

0.010

Log Frequency in Hz

15

f0

f1

f2

f3

2000

-6dB/Octave

Figure – 5: Endurance Vibration Profile for Fixed Wing Propeller Aircraft

Acceleration Spectral Density – g2/Hz

Frequency in Hz

10

1000

Figure – 6: Endurance Vibration Profile for Fixed Wing Jet Aircraft Aircraft

2000

0.001

0.01

0.1

0.040

20

-6 dB/Octave

### Final Resonance check

Final resonance search (similar to Initial Resonance search) shall be carried out after endurance test, even if there was no resonance found during initial resonance search.  Any deviation (more than 10%) in the resonances from the initial resonance search or any resonance noticed should be recorded and corrective action taken to eliminate the same. At the end of the vibration test, Visual Examination and Performance Check shall be carried out.

## CATH

This test shall be carried out to verify the capability of the equipment to meet the combined effect of the temperature, humidity and altitude.  The equipment shall be subjected to this combined environment as shown in Figure - 7 below for 01 (one) cycle.  High and Low Temperature (TOH, TOL, TSH, TSL) and Altitude (Max Alt) severity levels shall be decided based on the equipment specification / platform / field data.  The equipment shall be powered ‘ON’ during the test and Performance checks carried out as shown in profile. At the end of the test carry out Visual Examination and Performance Checks.

Note-6: The Combined Temperature / Humidity / Altitude test shown in Figure - 7 is for a typical fighter aircraft. For propeller aircraft/helicopters with very low Rate of Climb/Rate of Descent; the altitude, temperatures may be tailored based on the platform requirements and the requirement of combined high temperature/low pressure may be removed.

Note-7: Though, the combined Temperature / Humidity / Altitude test is the recommended method for avionics equipment, in case of non-availability of CATH facility, then temperature, altitude and humidity tests may be carried out separately as per the profiles given at Figure 8,9 10 & 11. (Figure shown constitute one cycle each). 03 such cycles each shall be carried out for high temperature and humidity test. One cycle each shall be carried out for low temperature and altitude test.

Figure – 7: Combined Temperature, Humidity and Altitude Test

60

TOL

60

30

60

30

60

Amb

43C

TOH

60

30

30

Time in minutes

Temperature

Altitude

Amb

MAX

ALT

Humidity

Amb

30%

95%

Performance  Checks

220

280

310

370

430

490

520

590

710

780

840

870

930

60

60

960

TSH

TSL

120

30

150

160

30

580

120

0

60

30

Power

ON

OFF

Note-8: All performance checks during last 30 minutes of the respective condition

Duration in minutes

10

10

30

720

## Shock

This test shall be carried out to verify the capability of the equipment to meet the mechanical shock requirements of the platform. The equipment shall be subjected to three shocks each of 20g saw tooth / 15g half sine pulse of 11 milliseconds duration in all six directions in ON condition. At the end of the shock test Visual Examination and Performance Check shall be carried out. The mounting arrangement / assembly shall be intact and equipment shall perform without degradation.

## Acceleration Test

The severity levels are as given below:

| ACCELERATION LEVEL | STRUCTURAL |  |
| --- | --- | --- |
|  | Aircraft | Helicopter |
| Fore | 3.0g | 4.0g |
| Aft | 9.0g | 4.0g |
| Lat (L) | 6.0g | 6.0g |
| Lat(R) | 6.0g | 6.0g |
| Up | 13.5g | 10.5g |
| Down | 4.5g | 4.5g |

- Directions with respect to the aircraft axis.

- If mounting orientation of the equipment in the aircraft is not known, test shall be done at maximum value in all directions.

- For other platforms Refer Mil-Std-810 or actual platform requirements.

Duration: One min dwell in each direction after following the centrifuge stabilization.

Direction: Three mutually perpendicular axes in two opposite directions along each axis.

Test Procedure:

- After the UUT is mounted on the acceleration test setup, bring the centrifuge to the speed required to induce the specified g level. Maintain this g level for at least one minute after the centrifuge rpm has stabilized.

- Stop the centrifuge and inspect the UUT for structural deformities/ cracks etc.

- Repeat steps 1 & 2 for the remaining five test directions

- Upon completing the tests in the six test directions, remove the UUT from the centrifuge and perform physical inspection and operational check.

Verify and record parameters in test sheets <Refer the test sheets >

## SHOCK (Crash Safety)

This test shall be carried out on cockpit and cabin mounted equipment to verify the capability of the equipment to withstand the Shock (Crash Safety). A mock-up of the equipment may be used for this test. The equipment shall be subjected to one shock each of 40g saw tooth / 30g half sine pulse of 11 milliseconds duration in all six directions in OFF condition. At the end of the shock test visual Examination shall be carried out. The mounting arrangement / assembly shall be intact

Direction:  Three mutually perpendicular axes in two opposite directions along each axis.

Test Procedure:

- Install the UUT on acceleration test setup and bring the centrifuge to the speed required to induce the specified g level above. Maintain this g level for at least one minute after the centrifuge rpm has stabilized.

- Stop the centrifuge and inspect the UUT

- Repeat this test procedure for the other test directions

- Upon completing the tests in the six test directions, inspect the UUT

Verify and record parameters in test sheets < Refer the test sheets >

## Rapid Decompression:

This test shall be carried out on equipment mounted in pressurised area such as cockpit / cabin to verify the capability of the equipment to meet the Rapid Decompression due to rapid change in pressure.

Test Procedure:

- With the UUT in the storage or transit configuration (Unit OFF), install it in the chamber and adjust the chamber air pressure  at a rate not to exceed  3°C/min (to the cabin altitude (2,438m (8,000 ft)

- Reduce the chamber air pressure to 18.8 kPa (2.73 psi) corresponding to 12,192m (40,000 ft) in not more than 15 seconds. Maintain this stabilized reduced pressure for at least 10 minutes. The equipment can be kept OFF during this period.

- Adjust the chamber air to standard ambient conditions using a pressure change rate not   greater than 10m/s, and a temperature change rate not to exceed 3°C/min

- Verify and record parameters in test sheets <Refer test sheets>

## Explosive Atmosphere Test:

An explosive test shall be carried out to determine the ability of the equipment to either operate in fuel – air atmosphere without causing ignition and / or for verifying containment of an explosive or burning reaction of encased unit. The test may be performed based on Method 511.4 of MIL-STD-810F.

Test setup:

For UUT thermal stabilization measurements, install thermocouples on the most massive functional part of the UUT, and two thermocouples attached to the inside the of test chamber to detect any temperature increase due to burning of the mixture.

Test Procedure:

Procedure I - Operation in an explosive atmosphere.

- With the UUT installed, seal the chamber and stabilize the UUT and chamber inner walls to the +71°C (±2 °C).

- Adjust the chamber air pressure to simulate the highest operating altitude of the UUT (not to exceed 40,000 ft) plus 2000 meters (6600 ft) to allow for introducing, vaporizing, and mixing the fuel with the air.

- Slowly   introduce the required volume of n-hexane into the test chamber as the   simulated altitude begins to drop.

- Circulate the test atmosphere and continue to reduce the simulated chamber altitude for at least three minutes to allow for complete vaporization of fuel and the development of a homogeneous mixture, and for the chamber pressure to reach the test altitude.

- At a pressure equivalent to 1000 m (3300 ft) above the test altitude, verify the potential explosiveness of the fuel-air vapor by attempting to ignite a sample of the mixture taken from the test chamber using a spark-gap device or glow plug ignition source with sufficient energy to ignite a 3.82 percent hexane mixture. If ignition does not occur, purge the chamber of the fuel vapor and repeat Steps 1-4. An alternative method of determining the explosive characteristics of the vapor is by using a calibrated explosive gas meter that   verifies the degree of explosiveness and the concentration of the fuel-air mixture.

- Although above the maximum operational altitude of the UUT, attempt to operate the UUT and continue operation from this step until completion of Step 8. Note the altitude at which the UUT begins proper operation.

- To ensure adequate mixing of the fuel and air, slowly decrease the simulated chamber altitude at a rate no faster than 100 meters per minute by bleeding air into the chamber.

- Stop decreasing the altitude at 1000 m below the test altitude or ground level, whichever is reached first, and perform operational check and switch off power to the UUT.

- Verify the potential explosiveness of the air-vapor mixture as in Step 5 above. If ignition does not occur, purge the chamber of the fuel vapor, and repeat the test from Step 1.

- Adjust the simulated chamber altitude to the equivalent of 2000 m above site pressure.

- Repeat Steps 3-7 at site pressure, perform operational check and switch-off power to the test item.

- Verify the potential explosiveness of the air-vapor mixture as in Step 5 above. If ignition does not occur, purge the chamber of the fuel vapor, and repeat the test from Step 10.

- Verify and record parameters in test sheets <give test sheet reference>

## Gunfire Vibration

Gunfire vibration tests are performed to provide a degree of confidence that materiel can physically withstand the transient vibration encountered in operational environments during the firing of a low calibre gun. This test shall be carried out only if the equipment is mounted near to the gun and gun firing is expected during the flight trials for this equipment. The profile may be tailored based on Method 519.5 of MIL-STD-810F. The Gun Fire Vibration Test for SOFT may be carried out on a Mock up unit.

## Final Ambient check:

This test is carried out after completion of all the environmental tests.

Test Procedure:

Switch on the UUT on the bench as per the Test Setup.  Perform the complete parameter check at 28.0VDC power supply under ambient temperature as per FTP and record the values.

After completion of all the environmental and EMI/EMC tests, compliance chart as given in the annexure III shall be prepared and submitted to RCMA along with SOFT results and application for SOF clearance of UUT.

Figure – 8: High Temperature Storage cum Operational Test Profile

10

20

30

40

50

60

70

6

9

11

21

12.5

14.5

16

18

24

Switch ON

Carry out PC during last 30 minutes

Temperature  ° C

Time in Hours

TOH

Amb

TSH

Figure – 9: Humidity Test Profile

0

2

8

16

24

30

60

95±4%

Time in Hours

Temperature °C

During temp decrease RH not less than 85%

Relative Humidity

Switch ‘ON” and PC during last 30 min of Final Cycle

PC & Switch OFF

PC After Recovery

3°C/minute

Switch ON & PC

2 hr

1 hr

1 hr

Amb

Time

TOL

TSL

Figure – 10: Low Temperature Storage cum Operational Test Profile

Temperature  ° C

PC & Switch OFF

PC After Recovery

Not more than 10 m/sec

2 Hours

Switch ON & PC during last 30 min

Amb Alt &  Pressure

Pressure of Max Alt  &

Temp corresponding to the Alt

Temp/ Pressure

Figure – 11: Altitude Test Profile

Time

# ANNEXURE-I: Test sheets

<All the test sheets/Test results recording format for the SOF tests are to be included>

Note: Test sheets should have the following data:

- LRU name, platform name and sl.no. of the UUT.

- Environmental Test name and part of the test profile in which parameters are being taken.

- Sl. no. of the UUT

- Software version & Checksum of the UUT

- Date and Venue of the test

- Input data

- Expected Output

- Observed Output

- Remarks (on test pass/ fail, test setup related observations and deviation from the procedure etc)

- Signatures of the test team including AQA Reps

ANNEXURE-II:  Compliance Chart

| Sl.No. | Type Test Schedule Clause No. | Name of the Test | Test Requirement | Compliance Status/ Remarks |
| --- | --- | --- | --- | --- |

(Designer)			(Design QC)			(Rep AQA)

Acronyms :

<Complete list of acronyms used in this document>